NETWORKING TAXWISE for TAX-AIDE

For Windows XP Professional or Windows 2000

(modified to allow for TrueCrypt Volumes and AVG Firewall)


Why?

Security:

· Only one computer contains taxpayer data.

Convenience:

· Only one computer needs TaxWise updates.

· Only one computer needs to be backed up.

· All connected computers have access to all returns.

· Printer sharing is easy – no printer switch boxes are required.

When?

Any time a site uses two or more computers up to a maximum of ten computers.

How?
The following pages show how to set up for networking.

———

[image: image9.jpg]


[image: image10.png]Wizards
can be
Hazardous
to your
TaxWise Network Health



There is a wealth of information in both Windows Help and on the Internet on how to set up networks. Virtually all the help assumes a basic “Home” network set up to provide shared access to the Internet from a few computers.

[image: image11.png]irewall Automatic Configuration Wizard 21x|

Existing configuration
Fiewall corfiguralion aeady evists

The settings aieady esist Do you want o update your curent corfiguration or generate
new setings and complelely replace your curtent configuation?

WARNING! Allyour custom stings willbe removed if you conlim cresting 3 new
configuation.

Requied operation
 Update curent setings

e 2 e sorfiaien

@ bk | Newss Cancel Erish




Windows also provides “Wizards” to help set up a home network with a focus on Internet access.

Networking for TaxWise has some different requirements.

Avoid using any of the Windows Networking Wizards.

Stick to the instructions listed below.

FULL WIRELESS NETWORKING of TAXWISE IS NOT PERMITTED or SUPPORTED BY THE IRS, UTS,  or TAX-AIDE.

WIRELESS NETWORKING TO SHARE A PRINTER IS ALLOWED. However SUPPORT is NOT AVAILABLE from UTS or the IRS and is LIMITED from TAX-AIDE
A network can have between two and ten computers. Generally there is one printer, but there can be two or more.

Two computers are connected using a Crossover Ethernet Cable. More than two are connected to a network router or a network switch using standard Ethernet Cables.

The most powerful computer is generally chosen for the TaxWise server if full networking is to be done.  (If possible, use a computer with XP-Pro, not XP-Home.)
Almost all the steps that need to be taken are in the computer setup. Be sure that the network is working before you attempt to configure or use TaxWise.

The TaxWise program can be already installed on the computers, or not.

Every computer connected in a network can be used for doing tax returns.

A network can be split into two or more sections if physical space is such that too many cables need to be run. Multiple network switches can be connected using a single Ethernet cable between each switch. Computers are connected to the switches as needed based on physical layout.

Part 1 of the instructions applies to all computers. 

Part 2 has instructions for connecting computers and configuring them just to share a printer.

Part 3 has instructions for full networking of TaxWise.

Appendix A has some equipment information together with some possible sources.

Appendix B has some troubleshooting information.

Appendix C presents “AVG Firewall Use and Configuration”.

Appendix D will have “Questions and Answers” (not yet completed).

PART 1 FOR ALL COMPUTERS

1. Windows accounts:
It is better, but not essential, for all computers connected on a LAN to have the same Windows account and password. If you have a good understanding of Windows Security and Networking then different accounts can be used. For TaxWise installation and operation the accounts must belong to the Administrators Group. Most IRS and AARP computers come set with accounts in the administrators group, however if in doubt check. 
Start > Settings > Control Panel > User Accounts. Each account will show which group it belongs to.

All IRS Depot supplied laptops are preset with the Windows account and password to be used this year. The account and password changes each year on IRS Depot supplied computers.

Windows accounts can be created if needed on either the AARP owned computers, or IRS Depot supplied. The quantity of each will indicate which ones to have accounts added so all units match.

Adding an account is done in Windows Control Panel – User Accounts. See Windows Help if you don’t know how to add an account.
 
IT IS MANDATORY THAT ALL WINDOWS ACCOUNTS HAVE A PASSWORD.

2. Computer Name and Workgroup Name.
All computers on a network must have a unique name, and belong to the same workgroup.

IRS Depot supplied laptops will have a unique name, but it’s not easy to remember – change it! Using the IRS Barcode Number is a good name choice, but anything can be used. Windows will tell you if you use an invalid character in the computer name.

The workgroup will be almost certainly be set to VOLGROUP. If it isn’t then change it to VOLGROUP (it is more secure than using the default Windows names)

To change computer name and/or workgroup:
Right click on “My Computer” on the Windows Desktop. Click on Properties. Click on the “Computer Name” tab. There is no need to enter anything in Computer description. Click on Change. Type the Computer name. Recommendation is that you use the Asset Tag Number (Barcode Number) for the computer. Prefix with a “T” if it is an AARP computer with an all-numeric asset number. Any name will do  – provided it is unique on the network. Make sure that the “Member of” section has Workgroup selected. Tab to the Workgroup field and enter VOLGROUP then click OK.

To make the changes take effect restart the computer.
3. Disk Write caching (not doing this step will not affect the ability to network the computers – this is a precautionary step to minimize possible problems later)
(This step will need to be done on Depot Computers)
(This step cannot be done on the new HP nx6310or HP nx6320  laptops which use a different hard drive interface. Turning off write caching is disabled on these computers).
Windows, by default, writes data to a disk through a disk cache (a temporary storage area in memory) to speed up overall disk access time. Under some circumstances due to a momentary failure of the network or a power source this can be a problem with networking and file sharing and could cause corruption of the TaxWise database.
To turn off the disk cache, double click on “My Computer”, then right click on Drive C: and select Properties. Click on the Hardware tab. Select the hard drive so it is highlighted blue and click Properties. Click on the Policies tab. Uncheck “Enable write caching on this disk”. Click OK twice then close My Computer. 
     NOTE – changes made to drive C will apply to any TrueCrypt volume 
     that is created on Drive C (drive P and S)

4. Windows Firewall (Windows XP only, not applicable to Windows 2000)
(This should not need to be done on Depot Computers – they are delivered with the Windows Firewall turned off)
The Windows Firewall Service must be running, but turned off. When the AVG Firewall is used then the Windows Firewall is automatically turned off. However the Windows Firewall Service itself must continue to run, as this controls other aspect of networking beyond just the Firewall. 
In the Control Panel double click on Windows Firewall. If the service is not running Windows will tell you. It does need to be started.  Select Off, even though it is not recommended, and click OK.
5. AVG Firewall
The AVG Firewall has great flexibility in protecting computers from unwanted access. The AVG firewall should be active at all times. However – as the typical use of AARP computers is on a private LAN the default setting for the AVG firewall must be “Computer in Domain”. Instructions on how to configure the AVG Firewall and set the default are contained in the document “AVG Firewall Use and Configuration” (see Appendix C to this document). 

6. Firewalls and Depot Laptops
IRS Depot supplied computers do not have Firewall software installed except for the built-in features of Windows XP. As previously noted the Windows firewall must be operational but turned off.  If AVG is installed on IRS Depot Laptops, then the IRS provided Norton anti-virus needs to be uninstalled.

If a Depot laptop is to be routinely connected to the Internet then either the Windows firewall must be activated for that connection time, or AVG software needs to be installed and set appropriately (see Appendix C). 
7. Network Connections.
(Some parts of this may need to be done on Depot Computers. It is not possible, in the image used by the IRS to configure their computer, to preset everything!)
In the Control Panel double click on “Network Connections”. Right click on “Local Area Connection” and select Properties. Be sure you are on the General Tab.
In the section “This connection uses the following items” make sure that every item is checked. You may have to scroll down to see them all.
Click once on “Internet Protocol (TCP/IP)” and click on Properties.
Make sure you are on the General Tab and check both “Obtain an IP address automatically”, and “Obtain DNS server address automatically”.  Click OK.  

8. Still in Local Area Connection Properties make sure that “Show icon in notification are when connected” is checked, and “Notify me when this connection has limited or no connectivity” is NOT CHECKED.  Then click OK.
Select Back to return to the Control Panel.

9. Folder Options.
Double click on Folder Options and select the “View” Tab.
Make sure that “Automatically search for network folders and printers” is NOT checked. (We know what we want; we don’t want windows to search for it).
Also make sure that “Hide extensions for known file types” IS NOT checked, and that “Use simple file sharing (Recommended)” IS NOT checked.
Note that Simple File Sharing is NOT a capability built into Windows 2000. Using Simple File Sharing does present a security risk – it is primarily provided for “home” networking and is the standard feature of Windows XP Home. In Windows XP Home Simple File Sharing is not changeable. However a computer running Windows XP Home should not be used as a server. It is acceptable for a workstation where file sharing is not required.
Click OK.
10. Power Options. The setting here will not stop networking, however we don’t want the server computer to go into “standby” or “hibernate”. This might happen if the server itself has no activity for a long time. It can also happen if running on batteries.
(Depot Computers will be set a little differently than AARP laptops The end result will be about the same EXCEPT for the “standby password” setting which you may want to change on Depot Computers).

Double click on Power Options. Under the “Power Schemes” tab set the Power scheme to Always On and click Apply.
Move to the Advanced tab. Make sure that “Prompt for password when computer resumes from standby” is NOT checked.
Move to the Alarms Tab. The settings when the computer was originally configured was for the Low battery alarm to trigger at 45% and to display a text warning. No other action.  The Critical battery alarm was set to activate at 20% and the computer to Hibernate.
It is NOT recommended to run a computer in a network on battery power. It may happen by “accident” and the existing warning should suffice. However – the Alarm settings and action can be changed if desired.  
Click Apply, then click OK.
Restart the computer and log on to the Windows account you will use.

PART 2 CONNECTING THE COMPUTERS AND SHARING A PRINTER.

1. Connect the Computers
If using just two computers connect them using a Crossover Ethernet cable.
If using more than two computers connect each of them to a router or network switch with standard Ethernet cables. Make sure the power supply for the network switch is plugged in.

NOTE: For convenience during the setup process everything can be done using just a crossover cable – one computer at a time – provided the “server computer” is always connected.

Depending on the make and model of the network switch used, a single light or a pair of lights should turn on for each connection.
Close to the Ethernet port on the computer a small, generally green, light should turn on. The light just means that the network cards and the switch are communicating. It does not mean that the network is actually complete.
2. Decide which computer will be the Print Server.
The computer with the printer directly connected to it using either a parallel printer cable, or a USB printer cable will be the Print Server. It can be the same computer as the computer designated as the TaxWise server (recommended) or a different computer on the network.

Some printers are “network ready” in that they can be connected using a network cable. For those familiar with networking this connection can be used. However it is recommended that the printer connection is either USB or Parallel.
If the printer driver needs to be installed on the Print Server computer do it now, following the manufacturers instructions, and physically connect the printer. The computer will recognize it. When done print a test page just to be sure.

The printer installation process will give the printer a generic name based on the printer model. It is better to give the printer a unique name so it is easy to recognize. If you have a network that has two HP 1022n printers installed it will be MUCH easier if they each have a unique name rather than both being called “HP LaserJet 1022n” or whatever the default name is. Names should be short and simple. For example LASER1, INKJET3, PRINT2, or something similar. To name the printer click on Start, move to Settings then Printers and Faxes. Click Printers and Faxes. Highlight the printer and right click on the name. Click on Rename and type the new name. Press the tab or enter key to finish the name change. Put a label on the printer showing it’s name (Please don’t write the name on the printer case with a marker pen, it’s almost impossible to get off without damaging the plastic).
3. Share the printer to the network.
Right click on the printer name you want to share and select “Sharing”. In the window that opens check “Share this printer”. The printer name will show automatically to whatever it was named in step 2.  Close Printers and Faxes.
4. Set all the other computers on the network to access the shared printer.

Before setting up the printer on all other computers the Firewall status must be checked on each computer. If AVG is installed the Filewall profile being used must be “Computer in Domain”. Open the AVG Control Center and look at the Firewall status. If it is not “Computer in Domain” then see Appendix C for how to configure the Firewall.

Do this on all the connected computers that do not have the printer directly connected.  Click Start, move to Settings then “Printers and Faxes”. Click on “Printers and Faxes”. Double click on “Add Printer”. The Add Printer Wizard will start (it’s OK to use this Wizard). Click Next. Select “A network printer, or a printer attached to another computer”. Click Next. “Browse for a printer” will be selected. Click Next.

You should see the network information, VOLGROUP, and under VOLGROUP the name of the Print Server Computer. Under that should be the name of the shared printer that you want to add. If a line of information has a tiny + sign double click on it to expand the information. You may need to wait a short while for the two computers to find each other and the printer. If you cannot see the printer double check the Firewall setting. Once you see the name of the printer highlight it and click Next. 

If you cannot find the printer on the network try shutting down and restarting the computers.  Always start the server first.

Windows will give a warning about printer drivers and viruses, just confirm that you want to go ahead. Then click “Finish”.
It is a good idea to print a test page from each computer. On each computer, right click on the printer, select properties, and “Print test page”.
IF YOU JUST WANT TO USE A NETWORK TO SHARE A PRINTER THEN STOP HERE.
Install TaxWise as a stand-alone program on each computer if it is not already installed. Set the printer in TaxWise Setup Options to be the network printer.

PART 3 Setting up TaxWise on the Network

Decide which computer will be the TaxWise Server. Generally this will be the most powerful computer, however local circumstances may dictate otherwise. The more computers linked in the network then the more power is required from the TaxWise server. Any computer can be used as the server should have at least 512 MB of ram, a processor speed of 1.0 GHz or more, and a fast hard drive.  HP nx6310 or HP nx6320 laptops are great – they have 1GB Memory and a fast hard drive. AARP nx61100 laptops are very adequate. Some IRS Depot computers are adequate.  Using a low powered server computer will not prevent networking, but overall network performance will be slow.

1. Share the TaxWise Server Disk Drive(s):

a) If the server computer is a Depot Laptop not using TrueCrypt.
In Windows Explorer locate Drive C: and right click on it.
Select Sharing and Security
You will see a Default Share that has been set by Windows as C$. This is for system purposes, and cannot be deleted.
Click on “New Share”
Enter a Share name of TWSRVR_C (recommended, but it can be any valid share name – just keep it short and meaningful).
Click on “Permissions” and check Allow Full Control for the “Everyone” user group. (Note – this will probably change for Tax Year 2007.)
Click Apply and then OK twice to close the “Share” Window and return to Windows Explorer.


b) If the server computer is using TrueCrypt for encryption:
Be sure the TrueCrypt is installed and operational with whatever procedure you have elected to use (Internal, External, or Traveller). 
NOTE: Use of Flash Drives as the main server drive is not recommended due to speed problems. Flash drives are not as fast as regular hard drives.

Make sure that the TrueCrypt volumes are mounted, but NOT shared for networking for this step. If Drives P and S are used this step will need to be done on both drives. 

Create a special “administrative share” for Drives P and S. Windows will do this automatically for Drive C, but cannot do it for Drives P and S as neither of them exist when Windows boots up. However the administrative share for Drive P and S will be retained once it is set.



The text below explains how to administratively share Drive P. 
If Drives Q, S or T are used just repeat the steps to each drive.

In My Computer or Windows Explorer right click on Drive P, select “Sharing and Security”.
In the window that opens check “Share this folder”. The share name will show P. Add a $ sign to make the share name P$.
Leave User limit checked as “Maximum allowed”
Click on “Permissions” .  Check “Allow Full Control” for the permissions. Allow Change will automatically become checked.  Click Apply and then OK, then Apply and OK again.

Repeat the above for Drive S, Q, and T if you are using them.

2. Map a Client (workstation) to the Server Drive.

If the server computer is an AARP machine using TrueCrypt click the TC_Start Icon and specify that this will be a server computer.


On any other one of the other computers right click on My Computer, and then click on Map Network Drive. A drive letter will be assigned by Windows – use the one supplied, or if your prefer you can use P (mapped to P on the server), S (mapped to S on the server) and so on. Don’t enter anything in Folder. Check Reconnect at Logon, and then click on Browse. In the “Browse for a folder” window click on the + signs to expand the entries. You will see VOLGROUP and the names of the computers connected. 

Click on the + sign alongside the name of the server computer. You will see the Server shared drive(s) TWSRVR_x (where x will be either C, P, R, or S). Click the + sign by TWSRVR_x  Click on TWSRVR_x and it will highlight blue. Click OK. Then click Finish.  A connection to TWSRVR_x will be created. A drive letter on the workstation is now “mapped” to the Server drive. Close the window.
If you are using both drives P and S on the Server (Two TrueCrypt volumes) then repeat the above steps for the other drive.

Repeat the above for all the connected computers – except the server.

3. Configure TaxWise on the Server.
(Note – if installing from a clone CD the procedure here might differ – however if a workstation number is not already assigned TaxWise will ask for one)

If TaxWise is not installed on the Server then install it now, just as usual from a TaxWise CD - except – when TaxWise asks “Is this a Network Installation” click Yes. TaxWise will require a Workstation Number. Enter 101 and click next. Installation will complete, but don’t run TaxWise yet – restart the computer.

When the computer has restarted start TaxWise. Go through the Setup process as usual except check the box “Save as Workstation Defaults”. Also, on the General Tab check “Network – My computer is a single or multi-user machine ATTACHED to others”. On the Printer Setup Tab select the Network Printer. Register TaxWise with your EFIN and Registration Code. TaxWise on the Server can be closed, or left running.

If TaxWise is already installed on the Server start TaxWise and log on as the Admin user. Go to Tools then Utilities/Setup Options and run through the Setup screens. On the General Tab check “Network” and also check the box  “Save as workstation defaults”. On the Printer Tab choose the network printer in all three boxes. If it is a local printer you will just see the printer name. If a printer connected to another computer you will see the printer name prefixed with the name of the computer it is attached to. Close TaxWise. 

Create the required Workstation number for TaxWise. Right click on My Computer, and click on Properties. Click on the Advanced Tab and then on Environment Variables. In the lower of the two Windows scroll down and see if the is an entry for Workstation. If there is then make sure it is 101. If there is no workstation entry click on “New” just below the lower window. For “Variable  name” enter Workstation, and for “Variable value” enter 101. Click OK three times, then restart the computer.

a) Setup and Configure TaxWise on the “Client” computers.


i) If TaxWise is already installed on any of the “Client” computers UTS recommend that it be uninstalled. It is possible to have TaxWise installed and run “stand-alone” on each of the client computers when that computer is not connected to a network. However when running on a network it is possible that a volunteer might start the “wrong version”. It must be a local decision as to whether to leave TaxWise installed on a network client computer.

ii) On any of the other computers double click on My Computer. You will see the Network Drive (for example TWSRVR_P (Z:). Double click on it. Move to the UTS06 Folder and double click. Move to the TWTECH folder and double click. Locate the SETUP.EXE program and double click on it. TaxWise workstation setup will start. You will be asked for a Workstation Number – Enter 102 for the first non-server computer; increment this number by one (1) for each successive computer installed on the same network (103,104, etc.). 

When the setup process is finished restart the computer. Once restarted there will be a desktop icon for TaxWise – TaxWise on Drive Z: (or whatever drive letter you chose). Double click on it and TaxWise will start.
APPENDIX A

Equipment:

Network Switch and/or Wireless Router:

8-Port Switch – Fast Ethernet (100Mbps).

Wireless Routers are generally 4-Port.

Many sources, prices vary every day! Check:

http://www.pricegrabber.com/search_attrib.php/page_id=17
Local sales (Office Depot, OfficeMax) can sometime get good prices.

You should not need to pay much more than $25

TrendNet and Zonet tend to be the lower cost makes. LinkSys and DLink are typically more expensive.

Refurbished items can often be bought with savings. Check out

http://www.tigerdirect.com/applications/Refurb/refurb_tlc.asp
Belkin has good wireless routers generally with a lifetime warranty.

Network adapters:

PCMCIA network adapters are readily available for laptops that do not have built-in Ethernet ports. Buy the ones that have an RJ45 connector, not one that has a “dongle”.

Shop around – Same sources as for network switches.

Network Cables:

Category 5 Ethernet. Not usually worth making your own. Mail Order is usually lowest cost. Places like Office Depot etc. are generally expensive. One good source is: http://store.pchcables.com/
Prices are very competitive, and shipment costs are reasonable.

APPENDIX B

If the network appears not to work be patient for a while. Even with all the settings correct it can take a while for Windows to “configure” the network. If you make a change to any settings there may be a delay in seeing them take effect. Modern computers are fast, but not instantaneous!!!

If – after re-checking all the setting in the instructions – you still cannot get networking to run and you are tearing your hair out then:

Windows Services.

It is possible that one of more of the Windows Services that are required for successful networking is disabled. 

To check the services click on Start, then Run. Enter “SERVICES.MSC”, and then click OK.

Verify that the following services ARE running:

Computer Browser

DHCP Client

DNS Client

Server

TCP/IP NetBIOS Helper

Workstation

With a default installation of Windows XP Pro or Home all the above should have a startup type of Automatic and therefore should be running. If they aren’t then change the settings.

Don’t change other services that look as though they might be involved!!!

Other problems

Even if you get a “green light” on every computer it is possible that a cable could be bad. Try a different cable.

Things that make you feel dumb.

· The network switch isn’t plugged in to it’s power adapter

· A Windows Firewall is running

· You are using a crossover cable where a regular one is required.

As a last resort

Send e-mail to TaxAideTech@aarp.org. Include your name, your address, your state, your position (e.g. LC, TC, TCS, etc.), and your telephone number.

Explain the problem in as much detail as you can – at least the make/model of the computers, and the operating system you are using. Be as specific as possible.  Someone will contact you and try to help.

APPENDIX C

AVG Firewall – it is important that the AVG Firewall is configured correctly to support TaxWise networking at a site. The following pages show how to configure the Firewall.

Computers that have the new AVG software installed are capable of networking even with the Firewall running. This is a major security improvement from prior years where any firewall generally had to be turned off.

The AVG Firewall is capable of tracking the type of network being used (called the Active Area in AVG) and changing it’s characteristics accordingly.

However in a private LAN typically used for Tax-Aide sites AVG can’t determine the type of network. There is no specific network controller. AVG will default to its default setting. The default setting when AVG is installed is “Computer on the move”. This is the most secure, and will stop any networking activity as file and printer sharing is NOT allowed by the firewall.

What follows describes how best to configure the AVG firewall for typical Tax-Aide use where networking IS a requirement.

AVG has three built-in Firewall profiles. Additional ones can be added, but this is not needed, and is beyond the scope of this document. The three provided are:

1. Standalone Computer. This would be a computer that is connected to the Internet via dial-up or direct connection to a Cable of DSL modem without a router being in place.
File and Printer sharing is NOT permitted with this profile.

2. Computer on the Move. A computer that is “mobile” and can connect to any available network. This is typical for laptops that are moving around and connecting to wireless “hot spots” or other unsecured networks.
File and Printer sharing is not permitted with this profile AND NEVER SHOULD BE!!

3. Computer In Domain. This is generally a computer on a controlled and managed network, but it also  applies to a Tax-Aide network where there is no connection to the Internet. Such a network is completely isolated from the outside world.


The following steps will configure the AVG Firewall Configuration from scratch. This is best done when the computer is not connected to a network of any kind.

Open the AVG Control Center. Your display may differ. If it does then click on View in the menu, and choose Reduced Mode  or press the F3 key.

[image: image1.png]| Companent. | status | Description
& artivius Internal Vius Database is up-to-date. Information sbout status and release date of the AVG Virus Database,
AntiSpywers  Anti-spyware is nstalled and fully functional, Protection against spyware, adware and tracking veb activty.

Protection against spam, e, unsolcted e-fas ith advertiserments.

€ Scheduer Next schedled task: 12/12/2006 11:16 AM Anki-Spam rules upd...  Automatic scheclled) friggering of Tests and Updates.

Resdent Shield  Resident Shild s loaded and Fuly functiona, Provids on-access scanning of executable fies and documents.
Virus Vault The tirus Vaul is empty. Virus quarantine, safe storage for infected fes.
€0 Update Manager  Last update on 12/12/2006 5:05 AM (today). Next update check...  Automatic AYG update from the Intermet or local netuwork.

Shel Extersion.  AUG i active n Windows Explrer. Antivius scanring i the Windows Explorer context men
E-mai Scanner  E-mai Scanner is fully functional, Scans incorting and outgeing e-mail messages.

License Your icense wil expire on 11/15/2008. When renswal is dus, ple...  Information about the AVG license.




Right Click on Firewall
[image: image2.png]Component Status Description
& artivius Internal Vius Database is up-to-date. Information sbout status and release date of the AVG Virus Database,
AntiSpywers  Anti-spyware is nstalled and fully functional, Protection against spyware, adware and tracking veb activty.

artiSpam antiSpam s nsealled and fuly Functonal, Protection againg spam, .. unsolted & mals with advertisements.
al — u compter

4 Scheder oorer 16 AM Anti-Spa udes upd.. Automatic(sheduld) tiggering o Tests and Ldates.
Residert Shied | CoMiaue cionl Provides on-access scanring of executableFles and documents.

4 virus Vaulk Show Log Virus quarantine, safe storage for infected fies,

€4 Update Manager  Stop Al Traffic

ShellExtension _ Deactivate Frewal

E-mail scanner
License

today). Next update check...  Automatic AVG update from the Internet or localnetwork.
Antivirus scaning n the Windows Explorer context menu.
Scans incoring and outgoing e-mail messagss.

When renewlis dus, pl...  Information about the AV license.





Run the Automatic Configuration Wizard:

	[image: image12.png]Network Connection Options
Howis your computer connected 1o the Itemnet?

For masimum secuty, without distubing the curret network services that you access, it s
necessaiy o select ALL conmection types that you use to connect t the Itemet.

Selectatleast e optian

¥ Standalon compute: - dectl connected o the Inteme.
¥ Computer in domain - connected to company network.

W Bl on he mave - when Tavelig

@ cchack | News Cancel Erish





	Make sure “Create a new configuration” is checked and click Next


	
	As a computer can be used with a variety of different connections to the Internet make sure that all three options are checked. 

Click Next.



	[image: image3.png]Scan for Intemet Applications ﬁ
‘Which areas of your computer should be scanned? L

Ta make sure your computer warks cortecty it i necessany to alow netork
commrication for same programs. The Frewal Automalic Carfiguration Wizard ill now
scan your computer and ty 1o find applications that should be lowed o commricale over
the network.

Select scanning option:
5
& Complete scan (scans al lcal hard dives of your computer]

" Selected aress scan (soans selected hard diives)

Dive Diive size]|_Free size] Dive label
©Cy 6B 15268 05
S\ 3968 29GB Documents

@ cchack |[Wain ]| concel





	Let the wizard do a complete scan of all drives (including but not necessary any mounted TrueCrypt volumes) to “find” known applications which will want to connect to the Internet. This is a fully automatic process.

Click Next and let the scan complete

	[image: image4.png]Firewall Automatic Cor 4 2x|

System Services ?
Whal system services and protocols do you use? o

Access s vill b created for al tems fsted below, accarding t the optians: allow, block.
The e species i the service o protocol can communicate over the network.
Cick on the items ioon to change i access optons.

% % L2TP VPN 2 pesndal
5-+es %= Microsoft file sharing and printing
% & Standalone computer e
'/ & Computer in domain W

5 25 Compteron the mave
= Novell NetWare Cliont
BT Ao, =l

[Allow to share Internet canneation with iher computers n the netwark

@ cchack | News Cancel Erish





	This display will show what the wizard has determined for system services.

Scroll down a little and find “Microsoft file sharing and printing”. Click on the “+” sign to expand the display and you will see that “file and printer sharing” is acceptable ONLY for a “Computer in a Domain”

This is as it should be.

Click Next on this screen and the following screen.


	[image: image5.png]irewall Automatic Configuration Wizard E 21x|

Select Current Profile ﬁ
Automatc Fiewall configuration has frished successiully e

The Firewall Automalic Conliguration Wizard s lead you thiough the configuation
process.

Il s have been created for selected system services and protocols, and for selected
pplcations commuricating over the network.

Selectthe profile you wauld ke to activate afer completing the wizard
" Standalone computes - dectly connected ta the Iteret
" Computerin damain - connected ta company netvrk.

€ Computer on the mave - when tiaveling

Press the Finish bultan o save al setings.

@ «Back [ Eane





	This display allows for the selection of the AVG firewall profile that will used when the wizard exits.

See below for the setting you should pick. Pick the one you want and click “Finish”

However there is a catch!!!! 

When the Wizard exits AVG will check to see if there is a network it can recognize. If it can’t find one then it will switch the Firewall Profile back to the default setting, which is probably “Computer on the move”.

The default setting cannot be changed in the Automatic Configuration Wizard. But – the default CAN be changed.



	If you are currently not connected to any network, or connected to a Tax-Aide LAN with no connection to the Internet then select “Computer in Domain”. HOWEVER – Depending on the actual network configuration AVG may quickly revert to it’s default setting as it cannot clearly identify the type of network.

If you are connected into a home network where you have a router (not a switch or a hub) between your computer and your cable or DSL modem then select “Computer in Domain”.

If you are connected to the Internet via dial-up or direct connection to a cable or DSL modem select “Stand-alone computer”

If you are connected via a public Internet access point (whether wired or wireless) then chose “Computer on the move”.




Once the Automatic Configuration Wizard is complete we can set the default Firewall Configuration.

In the Control Center right click on “Firewall” and select “Configure”

[image: image6.png]Component Status Description
Antivius Internal Vius Database is up-to-date. Information sbout status and release date of tF
Anti-Spyware s instaled and full unctonal, Protection against spyware, adware and tracki
Ainti-Spam i nstalled and fully Functional, Protection against spam, L. unsolcted &-mai

- ert 3 5
Properties 1:16 P Anki-Spam ules update. ‘Automatic (schedued) triggering of Tests and U
lunctonal, Provides onraccess scanning of executable fies

irus Vault ShowLog %

lpdate Manag  stop all Traffic
Shell Extznsior

Virus quarantine, safe storage o ifected fles
(today). Next update check schedbled on 12/27/200...  Automatc AVG updets from the Internet or oz
Antivirus scanring in the Windows Explorer cont
Scans incoring and outgoing e-mail messagss.
108, When renewal is due, please visit www.grisoft.com... Information about the AVG license.

Deactivate Firewal
Automatic Configuration Wizard





	[image: image7.png]Advanced seftings

Defauitprofie
T Disable dialog for assigning new area

T~ Disable displaying information about areas and profil che





	In the Firewall Configuration choose the “Profile Switch” tab.

Set the Default profile to “Computer in domain”.

This is the most used profile in a Tax-Aide networking environment.

Click Apply then OK

	Right click on Firewall again and select “Properties”

	[image: image8.png]Setgs | Gons|

Firewal status-

Fiewal i unning and fully functionsl.
Fiewal i corfigured ok

Fiewall uprime: 028623
Blocked packets: 0
Total packets: 10867

Select profle (current: Computer o the movel:

Compui n domain o] us PnT%e

The Configue bulton apens the Fiewal

confguraion ddog Conligurs

Deactivate Fiewal Stop Al Traffic

[ Cancel Help





	In “select profile” make sure “Computer in Domain” is chosen, and click on “Use Profile”


AVG will now use the default of “Computer in Domain” whenever it cannot tell what type of network is operational.

If a different kind of network is detected – wireless or wired – AVG will recognize that and ask what profile should be assigned to this network. Generally on a wireless network “Computer on the move” should apply. On a wired network then Computer in Domain if the network is secure, or Stand-alone computer if the network is not secure (computer connected directly to cable modem for example).

Once AVG has been “taught” the basics it will respond accordingly an no further action is required.

HOWEVER for work at certain Tax-Aide sites:

When a computer is in use at a Tax-Aide site with a secure private LAN the “Computer in Domain” will be used (file and printer sharing allowed). 

If the server computer on that LAN is then connected to the Internet to transmit returns the firewall profile must be changed to “Computer on the move”. File and printer sharing will not be permitted. This provides an extra level of protection from outside sources. The change is quick and easy to do in the Control Center by changing the profile in use

APPENDIX D

QUESTIONS AND ANSWERS

(This appendix will be completed later.)

X








NETWORKING TAXWISE for TAX-AIDE

Page 1 of 17
National Technology Committee                         Updated 3/19/2007



